WATER QUALITY DATA

The table below lists all the contaminants that we detected during the 2021 calendar year. The presence of these contaminants in the water does not necessarily indicate that the water poses a health risk. Unless otherwise indicated, the data presented in this table is from testing done between January 1 and December 31, 2021. The Indiana Department of Environmental Management (IDEM) requires us to monitor certain contaminants at a frequency less than once per year because the concentrations of these contaminants are not expected to vary significantly from one year to another. Some of the data, though representative of the water quality, may however be more than one year old.

Some of the terms and abbreviations used in this report are:

MCL: MAXIMUM CONTAMINANT LEVEL, THE HIGHEST LEVEL OF A CONTAMINANT THAT IS ALLOWED IN DRINKING WATER.

MCLG: MAXIMUM CONTAMINANT LEVEL GOAL, THE LEVEL OF A CONTAMINANT IN DRINKING WATER BELOW WHICH THERE IS KNOWN OR EXPECTED RISK TO HEALTH.

MRDL: MAXIMUM RESIDUAL DISINFECTANT LEVEL GOAL, THE HIGHEST LEVEL OF DISINFECTANT ALLOWED IN DRINKING WATER.

MRDLG: MAXIMUM RESIDUAL DISINFECTANT LEVEL GOAL, THE LEVEL OF DRINKING WATER DISINFECTANT BELOW WHICH THERE IS NO KNOWN OR EXPECTED RISK TO HEALTH.

AL:  ACTION LEVEL, THE CONCENTRATION OFA CONTAMINANT WHICH, WHEN EXCEEDED, TRIGGERS TREATMENT OR OTHER REQUIREMENTS OR ACTION WHICH A SYSTEM MUST FOLLOW.

TT: TREATMENT TECHNIQUE A REQUIRED PROCESS INTENDED TO REDUCE THE LEVEL OF A CONTAMINANT IN DRINKING WATER.

NTU: NEPHELOMETRIC TURBIDITY UNIT, A MEASURE OF THE CLARITY (OR CLOUDINESS) OF WATER.

PPM: PARTS PER MILLION, A MEASURE FOR CONCENTRATION EQUIVALENT TO MILLIGRAMS PER LITER.

PPB: PARTS PER BILLION, A MEASURE FOR CONCENTRATION EQUIVALENT TO MICROGRAMS PER LITER.

PCI/L; PICOCURIES PER LITER, A MEASURE FOR RADIATION.

E_: POTENTIAL VIOLATION, ONE THAT IS LIKELY TO OCCUR IN THE NEW FUTURE, SUBJECT TO OTHER APPLICABLE REQUIREMENTS.

!:Y_A: EITHER NOT AVAILABLE OR NOT APPLICABLE.

ND:  NOT DETECTED, THE RESULT WAS NOT DETECTED AT OR ABOVE THE ANALYTICAL METHOD DETECTION LEVEL.

OUR WATERSHED PROTECTION EFFORTS Our water system is working with the community to increase awareness of better waste disposal practices to further protect the sources of our drinking water. We are also workings with other agencies and with local watershed groups to educate the community on ways to keep our water safe.

PUBLIC INVOLEMENT OPPORTUNITIES Our Water Board Meetings are held on the first Wednesday of the each month at 6:00pm at Springs Valley Water's office, located at 7959 W Hunters Run Dr. in French Lick, IN.

PLEASE SHARE THIS INFORMATION Large water volume customers (like apartment complexes, hospitals, schools, and/or industries) are encouraged to post extra copies of this report in conspicuous locations or to distribute them to your tenants, residents, patients, students, and/or employees. This "good faith" effort will allow non-billed customers to learn more about the quality of the water that they consume.

Annual Water Quality Report for the period of January 1 to December 31, 2021

This report is intended to provide you with important information about your drinking water and the efforts made by the water system to provide safe drinking water.

For more information regarding this report contact:

Name Anthony Galloway Phone 812-936-9658

PWSID: INS259002	Springs Valley Regional Water District	page 2 of 3

# 2021	Regulated Contaminants Detected

Lead and Copper

Definitions:

Action Level Goal (ALG): The level of a contaminant in drinking water below which there is no known or expected risk to health. ALGs allow for a margin of safety. Action Level: The concentration of a contaminant which, if exceeded, triggers treatment or other requirements which a water system must follow.

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home plumbing. We are responsible for providing high quality drinking water, but we cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at

| Lead and Copper | Date Sampled | MCLG | Action Level (Al) | 90th Percentile | # Sites Over Al | Units | Violation | Likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Copper | 2021 | 1.3 | 1.3 | 0.0152 | 0 | ppm | N | Erosion of natural deposits; Leaching from wood preservatives; Corrosion of household plumbing systems. |

Regulated Contaminants

| Disinfectants and Disinfection By-Products | Collection Date | Highest Level Detected | Range of levels Detected | MCLG | MCL | Units | Violation | likely Source of Contamination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chlorine | 2021 | 1 | 1-Jan | MRDLG =4 | MRDL=4 | ppm | N | Water additive used to control microbes. |
| Haloacetic Acids (HAA5) | 2021 | 28 | 21 - 35.5 | No goal for the total | 60 | ppb | y | By-product of drinking water disinfection. |
| Total Trihalomethanes (TTHM) | 2021 | 45 | 21 - 72 | No goal for the total | 80 | ppb | y | By-product of drinking water disinfection. |

Violation Tables

| Total Trihalomethanes {TTHM) |  |  |  |
| --- | --- | --- | --- |
| Some people who drink water containing trihalomethanes in excess of the MCL over many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased riskI of getting cancer. |  |  |  |
| Violation Type | Violation Bee:in | Violation End | Violation Explanation I |
| MONITORING, ROUTINE (DBP), MAJOR | 10/1/2021 | 12/31/2021 | We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our I drinking water during the period indicated. |

| Haloacetic Acids {HAAS) |  |  |  |
| --- | --- | --- | --- |
| Some people who drink water containing haloacetic acids in excess of the MCL over many years may have an increased risk of getting cancer. I |  |  |  |
| Violation Type | Violation Beein | Violation End | Violation Explanation I |
| MONITORING, ROUTINE (DBP), MAJOR | 10/1/2021 | 12/31/2021 | We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our I drinking water during the period indicated. |

PWSID: IN5259002	Springs Valley Regional Water District                                                                                           page 1 of 3

| 2021 Monitoring Results for Patoka Lake Regional Water & Sewer District |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Some people may be more vulnerable to contaminants in drinking water than the general population. [mmuno- compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/A[DS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. EPA/CDC guidelines on appropriate means to lessen the risks of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline at 800) 426-4791. |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSTITUENTS | Date Tested |  | Unit | MCL |  | MCLG |  | MRAA | Range | Violation | Major Sources |  |
| DIS£NFECTION PROCESS BYPRODUCTS |  |  |  |  |  |  |  |  |  |  |  |  |
| HAAS's (Total Haloacetic Acids) | 2021 | Ppb |  | 60 - |  | NA |  | 34.9 | 25 TO45 | No | Disinfection process byproduct |  |
| TTHM'S (Total Trihalomcthanes} | 2021 | Ppb |  | 80 |  | NA |  | 41.7 | 20.4 TO 60.9 | No | Disinfection process byproduct |  |
| INORGANIC CONSTITUENTS |  |  |  |  |  |  |  |  |  |  |  |  |
| Fluoride | 2021 | Ppm |  | 4 |  | 4 |  | .6 |  | No | Water additive to promote strong teeth & erosion of nab.Jral .deoosits |  |
| Copper | 2020 | µg/L |  | 1300 AL |  |  |  | 170 | 90"' percentile value | No | Corrosion of household olumbin2 |  |
| Lead | 2020 | µg/L |  | ISA! |  |  |  | 3.7 | 9Qlh percentile value | No | Corrosion of household olumbin2. |  |
| (For Lead & Coooer the number of samoles above AL is 0.l |  |  |  |  |  |  |  |  |  |  |  |  |
| Sodium | 2021 | PPM |  | None |  | None |  | 2.9 | NA | No | Erosion of natural deposits |  |
| Atrazine | 2021 | Ppb |  | 3.0 |  | BDL |  | BDL | NIA |  |  |  |
| Barium | 2021 | PPM |  | 2 |  | BDL |  | 0.Q25 | NIA | No | Erosion of natural deposits |  |
| EPA is preparing a regulation, which wiil specify a Maximum Contaminant level for radon. Radon is a radioactive gas that occurs naturally in ground water and is released from water into the air during household use. At high exposure levels it can cause lung cancer. Radon was not detected in the treated finished water distributed bv Patoka Lake Regional Water & Sev.rer District. |  |  |  |  |  |  |  |  |  |  |  |  |
| Gross Alpha | 2020 | pCi/L |  | 15 |  | 0 |  | 1.7 | NIA | No | Runoff from herbicide used on row croos |  |
| Radium 226 | 2016 | pCi/L |  |  |  | 0 |  | 0.\4 | NIA | No | Erosion of natural deoosits |  |
| Radium 228 | 2020 | pCi/L |  |  |  | 0 |  | (l:t7 | NIA | No | Erosion of natural· deoosits |  |
| Combined Radium | 2016 | pCi/L |  | 5 |  | 0 |  | .97 | NIA | No | Erosion of natural deoosits |  |
| Turbidity | Daily | NTU |  | TT,;Q.3 |  | NA |  | .25 | Highest reading | No |  |  |
| Turbidity does not present any risk to your health. Turbidity is a measure of suspended matter in water. and is a good indicator that the filtration . svstem is functionini;t. |  |  |  |  |  |  |  |  |  |  |  |  |
| TOTAL ORGANIC CARBON |  |  |  |  |  |  |  |  |  |  |  |  |
| Average percent of removal |  | % |  |  | 25% |  | 100 | 34% | 27.9% TO 40.5% | No | Erosion of natural deposits |  |
| UN REGULATI ED COI NTAMlNANTS- |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSTITUENTS Date Unit Tested |  |  |  |  | MRDL |  | MRDLG | MRAA | Range | Violation | Major Sources |  |
| Chloramine I Dailv I Porn |  |  |  |  | 4.0 |  | 4.0 | 3.63 | 3.9J 10 2.9 I | No | Added for disinfectant |  |
| - ,, |  |  |  |  |  |  |  |  |  |  |  |  |

EXPLANATION OF THE WATER QUALITY DATA TABLE

This report is based upon tests performed by Patoka Lake Regional Water & Sewer District personnel and contracted labs. Terms used in the Water Quality Table' and in other parts of this report are defined here.	·

Definitions:

IDEM- Indiana Department ofEnvironmental Management

EPA - Environmental Protection Agency

MCL- Maximum Contaminant Level: The highest level of a contaminant that is allowed in drinking water as established by EPA. The MCL's are set as low to the MCLG's as feasible using the best available treatment technology.

MCLG - Maximum Contaminant Level Goal: The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLG's allow for a margin of safety.

AL- Action Level: The concentration of a contaminant, which, if exceeded, trigger treatment or other requirements that a water system must follow.

TT - Treatment Technique: A required process intended to reduce the level of a contaminant in drinking water.

MRDL-Maximum Residual Disinfectant Level

MRDLG - Maximum Residual Disinfectant Level Goal

NTU = nephelometric turbidity units pCi/L = picocurie per liter

MRAA = maximum running annual average

Key to Table

VOC = volatile organic contaminants

ppm= parts per million, or milligrams per liter (mg/I) ppb = parts per billion, or micrograms per liter (µg/L)

CHLORAMINES:

Note: Since 1983, the District has used chloramines to qisinfect your drinking water. For all normal users, chloraminated water is the same as water disinfected with chlorine. However, kidney dialysis patients and aquarium or fish pond owners need to take special precautions when using chloraminated water. Kidney dialysis patients should consult your doctors and fish owners should call your pet store for more information.

Statement Addressing Lead in Drinking Water:

If present, elevated levels oflead can cause serious health problems, especially for pregnant women and young children. Lead in drinking water is primarily from materials and components associated with service lines and home_plumbing. Patoka Lake Regional Water & Sewer District is responsible for providing higlt'quality "drinking water, but cannot control the variety of materials used in plumbing components. When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. If you are concerned about lead in your water, you may wish to have your water tested. Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

PWSID: INS259002	                                            Springs Valley Regional Water District	                                                                                       page 2 of 3